PRESTO NEO® DUO – 64767

Single wall-mounted self-closing tap, with an ergonomic soft touch lever to control water flow, 1/2", hot or cold water. As PRESTO NEO® DUO ref.64767 or technically equivalent. Equipped against corrosion and lime scale with a self-cleaning needle and ruby mechanism, preset at 3 l/mn. 7 sec timed-flow (lever upwards or downwards). Flow rate and flow time could be changed by internal device only. NF compliant.

Or

Single wall-mounted self-closing tap, with an ergonomic soft touch lever to control water flow, 1/2", hot or cold water. NF compliant. As PRESTO NEO® DUO ref.64767 or technically equivalent. Including:

- A self-cleaning needle and ruby mechanism

- Preset at 3 l/mn flow rate (inner adjustment)

- 7 sec timed-flow (lever upwards or downwards)

- Flow rate and flow time could be changed by internal device only

- Soft touch

.